ES8311

# Low Power Mono Audio CODEC

# FEATURES

System

- High performance and low power multi- bit delta-sigma audio ADC and DAC

- I2S/PCM master or slave serial data port

- 256/384Fs, USB 12/24 MHz and other non standard audio system clocks

- I2C interface ADC

- 24-bit, 8 to 96 kHz sampling frequency

- 100 dB signal to noise ratio, -93 dB THD+N

- One pair of analog input with differential input option

- Low noise pre-amplifier

- Noise reduction filters

- Auto level control (ALC) and noise gate

- Support analog and digital microphone

DAC

- 24-bit, 8 to 96 kHz sampling frequency

- 110 dB signal to noise ratio, -80 dB THD+N

- One pair of analog output with headphone driver and differential output option

- Dynamic range compression

- Pop and click noise suppression Low Power

- 1.8V to 3.3V operation

- 14 mW playback and record

- Low standby current

# APPLICATIONS

  - Automotive

  - Phone

  - Toy

  - 2-way radio

  - Dash cam

  - IP Camera

  - DVR, NVR

  - Surveillance

# ORDERING INFORMATION

ES8311 -40°C ~ +105°C QFN-20

1

DSDIN

ASDOUT

SCLK LRCK

CDATA

CCLK CE

MCLK

BLOCK DIAGRAM

PGA

Mono

ADC

Mono

DAC

HP Driver

Power Supply

Analog Reference

ADC ALC DAC DRC

Noise Filter

I2C

I2S/PCM

Clock Mgr

DVDD

PVDD DGND AVDD AGND

ADCVREF

DACVREF VMID

MIC1P MIC1N

OUTP OUTN

VMID

MIC1N MIC1P/DMIC_SDA

CDATA

CE

PIN OUT AND DESCRIPTION

1

2

3

4

5

ES8311

15

14

13

12

11

16

17

18

19

20

10

9

8

7

6

AGND

DSDIN LRCK ASDOUT

SCLK/DMIC_SCL

CCLK MCLK PVDD DVDD DGND

ADCVREF DACVREF OUTN OUTP AVDD

| Pin Name | Pin number | Input or Output | Pin Description |
| --- | --- | --- | --- |
| CCLK, CDATA, CE | 1, 19, 20 | I, I/O, I | I2C clock, data, address |
| MCLK | 2 | I | Master clock |
| SCLK/DMIC_SCL | 6 | I/O | Serial data bit clock/DMIC bit clock |
| LRCK | 8 | I/O | Serial data left and right channel frame clock |
| ASDOUT | 7 | O | ADC serial data output |
| DSDIN | 9 | I | DAC serial data input |
| MIC1P/DMIC_SDA MIC1N | 18 17 | I | Mic input |
| OUTP, OUTN | 12, 13 | O | Differential analog output |
| PVDD | 3 | Analog | Power supply for the digital input and output |
| DVDD, DGND | 4, 5 | Analog | Digital power supply |
| AVDD, AGND | 11, 10 | Analog | Analog power supply |
| VMID | 16 | Analog | Filtering capacitor connection |
| ADCVREF, DACVREF | 15, 14 | Analog | Filtering capacitor connection |

DACVREF ADCVREF

VMID

AGND PGND

AVDD

GND(SYS)

0R

In the layout, chip is treated as a analog device

AGND

AGND

VA

AGND AGND

1uF 1uF 1uF

* * *

* 1uF

AGND

VD VP

*	*

0.1uF 0.1uF

4

3

5

MCU/DSP

AGND2

6

8

7

9

1

19

20

DVDD PVDD

DGND MCLK

SCLK/DMIC_SCL LRCK

ASDOUT DSDIN

CCLK CDATA CE

MIC1P/DMIC_SDA

MIC1N

18

17

1uF

1uF

MIC1P

MIC1N

ES8311

OUTN

13

OUTP   12

1uF

1uF

OUTN

OUTP

For the best performance,decoupling and filtering capacitors should be located as close to the device package as possible

* Additional paralle capacitors(typically 0.1 μF) can be used, larger value capacitors(typically 10 μF) would also help

14

15

16

10

21

11

TYPICAL APPLICATION CIRCUIT

- CLOCK MODES AND SAMPLING FREQUENCIES

The device supports standard audio clocks (64F, 128Fs, 256Fs, 384Fs, 512Fs, etc), USB clocks (12/24 MHz), and some common non standard audio clocks (16 MHz, 25 MHz, 26 MHz, etc).

According to the serial audio data sampling frequency (Fs), the device can work in two speed modes: single speed mode or double speed mode. In single speed mode, Fs normally ranges from 8 kHz to 48 kHz, and in double speed mode, Fs normally range from 64 kHz to 96 kHz.

The device can work either in master clock mode or slave clock mode. In slave mode, LRCK and SCLK are supplied externally, and LRCK and SCLK must be synchronously derived from the system clock with specific rates. In master mode, LRCK and SCLK are derived internally from device master clock.

# MICRO-CONTROLLER CONFIGURATION INTERFACE

The device supports standard I2C micro-controller configuration interface. External micro- controller can completely configure the device through writing to internal configuration registers.

I2C interface is a bi-directional serial bus that uses a serial data line (CDATA) and a serial clock line (CCLK) for data transfer. The timing diagram for data transfer of this interface is given in Figure 1a and Figure 1b. Data are transmitted synchronously to CCLK clock on the CDATA line on a byte-by-byte basis. Each bit in a byte is sampled during CCLK high with MSB bit being transmitted firstly. Each transferred byte is followed by an acknowledge bit from receiver to pull the CDATA low. The transfer rate of this interface can be up to 400 kbps.

A master controller initiates the transmission by sending a “start” signal, which is defined as a high-to-low transition at CDATA while CCLK is high. The first byte transferred is the slave address. It is a seven-bit chip address followed by a RW bit. The chip address must be 0011 00x, where x equals CE. The RW bit indicates the slave data transfer direction. Once an acknowledge bit is received, the data transfer starts to proceed on a byte-by-byte basis in the direction specified by the RW bit. The master can terminate the communication by generating a “stop” signal, which is defined as a low-to-high transition at CDATA while CCLK is high.

In I2C interface mode, the registers can be written and read. The formats of “write” and “read” instructions are shown in Table 1 and Table 2. Please note that, to read data from a register, you must set R/W bit to 0 to access the register address and then set R/W to 1 to read data from the register.

Table 1 Write Data to Register in I2C Interface Mode

|  | Chip Address | R/W |  | Register Address |  | Data to be written |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| start | 0011 00 CE | 0 | ACK | RAM | ACK | DATA | ACK | Stop |

Chip Addr

Write ACK

Reg Addr

ACK

Write Data

ACK

bit 1 to 7

bit 1 to 8

bit 1 to 8

CDATA

CCLK

START	STOP

Figure 1a I2C Write Timing

Table 2 Read Data from Register in I2C Interface Mode

|  | Chip Address | R/W |  | Register Address |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Start | 0011 00 CE | 0 | ACK | RAM | ACK |  |
|  | Chip Address | R/W |  | Data to be read |  |  |
| Start | 0011 00 CE | 1 | ACK | Data | NACK | Stop |

Chip Addr

Write ACK

Reg Addr

ACK

Chip Addr

Read ACK

Read Data

NO ACK

bit 1 to 7

bit 1 to 8

bit 1 to 7

bit 1 to 8

CDATA

CCLK

START

START

STOP

Figure 1b I2C Read Timing

# DIGITAL AUDIO INTERFACE

The device provides many formats of serial audio data interface to the input of the DAC or output from the ADC through LRCK, SCLK and DSDIN or ASDOUT pins. These formats are I2S, left justified, right justified and DSP/PCM. DAC input DSDIN is sampled by the device on the rising edge of SCLK. ADC data is out at ASDOUT on the falling edge of SCLK. The relationship of SDATA (DSIN/ASDOUT), SCLK and LRCK with these formats are shown through Figure 2a to Figure 2d.

1 SCLK	1 SCLK

L Channel

R Channel

LRCK

SCLK

SDATA

MSB	LSB	MSB	LSB

Figure 2a I2S Serial Audio Data Format

L Channel

R Channel

LRCK

SCLK

SDATA

MSB	LSB	MSB	LSB

Figure 2b Left Justified Serial Audio Data Format

1 SCLK

L Channel

R Channel

LRCK

SCLK

SDATA

MSB	LSB MSB	LSB

Figure 2c DSP/PCM Mode A Serial Audio Data Format

L Channel

R Channel

LRCK

SCLK

SDATA

MSB	LSB MSB	LSB

Figure 2d DSP/PCM Mode B Serial Audio Data Format

# ELECTRICAL CHARACTERISTICS

## ABSOLUTE MAXIMUM RATINGS

Continuous operation at or beyond these conditions may permanently damage the device.

| PARAMETER | MIN | MAX |
| --- | --- | --- |
| Analog Supply Voltage Level | -0.3V | +3.6V |
| Digital Supply Voltage Level | -0.3V | +3.6V |
| Analog Input Voltage Range | AGND-0.3V | AVDD+0.3V |
| Digital Input Voltage Range | DGND-0.3V | PVDD+0.3V |
| Operating Temperature Range | -40C | +105C |
| Storage Temperature | -65C | +150C |

## RECOMMENDED OPERATING CONDITIONS

| PARAMETER | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- |
| DVDD | 1.6 | 3.3 | 3.6 | V |
| PVDD | 1.6 | 3.3 | 3.6 | V |
| AVDD | 1.7 | 3.3 | 3.6 | V |

ADC ANALOG AND FILTER CHARACTERISTICS AND SPECIFICATIONS

Test conditions are as the following unless otherwise specify: AVDD=3.3V, DVDD=3.3V, AGND=0V, DGND=0V, Ambient temperature=25C, Fs=48 KHz, MCLK/LRCK=256.

| PARAMETER | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- |
| ADC Performance |  |  |  |  |
| Signal to Noise ratio (A-weigh) | 95 | 100 | 102 | dB |
| THD+N | -95 | -93 | -85 | dB |
| Gain Error |  |  | ±5 | % |
| Filter Frequency Response – Single Speed |  |  |  |  |
| Passband | 0 |  | 0.4535 | Fs |
| Stopband | 0.5465 |  |  | Fs |
| Passband Ripple |  |  | ±0.05 | dB |
| Stopband Attenuation | 70 |  |  | dB |
| Filter Frequency Response – Double Speed |  |  |  |  |
| Passband | 0 |  | 0.4167 | Fs |
| Stopband | 0.5833 |  |  | Fs |
| Passband Ripple |  |  | ±0.005 | dB |
| Stopband Attenuation | 70 |  |  | dB |
| Analog Input |  |  |  |  |
| Full Scale Input Level |  | AVDD/3.3 |  | Vrms |
| Input Impedance |  | 6 |  | KΩ |

## DAC ANALOG AND FILTER CHARACTERISTICS AND SPECIFICATIONS

Test conditions are as the following unless otherwise specify: AVDD=3.3V, DVDD=3.3V, AGND=0V, DGND=0V, Ambient temperature=25C, Fs=48 KHz, MCLK/LRCK=256.

| PARAMETER | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- |
| DAC Performance |  |  |  |  |
| Signal to Noise ratio (A-weigh) | 100 | 110 | 115 | dB |
| THD+N | -85 | -80 | -75 | dB |
| Gain Error |  |  | ±5 | % |
| Filter Frequency Response – Single Speed |  |  |  |  |
| Passband | 0 |  | 0.4535 | Fs |
| Stopband | 0.5465 |  |  | Fs |
| Passband Ripple |  |  | ±0.05 | dB |
| Stopband Attenuation | 53 |  |  | dB |
| Filter Frequency Response – Double Speed |  |  |  |  |
| Passband | 0 |  | 0.4167 | Fs |
| Stopband | 0.5833 |  |  | Fs |
| Passband Ripple |  |  | ±0.005 | dB |
| Stopband Attenuation | 56 |  |  | dB |
| Analog Output |  |  |  |  |
| Full Scale Output Level |  | AVDD/3.3 |  | Vrms |

## DC CHARACTERISTICS

| PARAMETER | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- |
| Normal Operation Mode |  |  |  |  |
| DVDD=1.8V, PVDD=1.8V, AVDD=3.3V |  | 8 |  | mA |
| Power Down Mode |  |  |  |  |
| DVDD=1.8V, PVDD=1.8V, AVDD=3.3V |  | 0 |  | uA |
| Digital Voltage Level |  |  |  |  |
| Input High-level Voltage | 0.7*PVDD |  |  | V |
| Input Low-level Voltage |  |  | 0.5 | V |
| Output High-level Voltage |  | PVDD |  | V |
| Output Low-level Voltage |  | 0 |  | V |

SERIAL AUDIO PORT SWITCHING SPECIFICATIONS

| PARAMETER | Symbol | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| MCLK frequency |  |  | 51.2 | MHz |
| MCLK duty cycle |  | 40 | 60 | % |
| LRCK frequency |  |  | 200 | KHz |
| LRCK duty cycle |  | 40 | 60 | % |
| SCLK frequency |  |  | 26 | MHz |
| SCLK pulse width low | TSCLKL | 15 |  | ns |
| SCLK Pulse width high | TSCLKH | 15 |  | ns |
| SCLK falling to LRCK edge | TSLR | –10 | 10 | ns |
| SCLK falling to SDOUT valid | TSDO | 11 |  | ns |

Figure 3 Serial Audio Port Timing

## I2C SWITCHING SPECIFICATIONS

| PARAMETER | Symbol | MIN | MAX | UNIT |
| --- | --- | --- | --- | --- |
| CCLK Clock Frequency | FCCLK |  | 400 | KHz |
| Bus Free Time Between Transmissions | TTWID | 1.3 |  | us |
| Start Condition Hold Time | TTWSTH | 0.6 |  | us |
| Clock Low time | TTWCL | 1.3 |  | us |
| Clock High Time | TTWCH | 0.4 |  | us |
| Setup Time for Repeated Start Condition | TTWSTS | 0.6 |  | us |
| CDATA Hold Time from CCLK Falling | TTWDH |  | 900 | ns |
| CDATA Setup time to CCLK Rising | TTWDS | 100 |  | ns |
| Rise Time of CCLK | TTWR |  | 300 | ns |
| Fall Time CCLK | TTWF |  | 300 | ns |

Figure 4 I2C Timing

# PACKAGE

- CORPORATE INFORMATION

Everest Semiconductor Co., Ltd.

No. 1355 Jinjihu Drive, Suzhou Industrial Park, Jiangsu, P.R. China, Zip Code 215021

苏州工业园区金鸡湖大道 1355 号国际科技园, 邮编 215021 Email: info@everest-semi.com